10 73 00 SECTION

Trellis and Pergola

- GENERAL

  - SECTION INCLUDES

    - Pre-engineered, pre-finished  Pergola system.

    - Pre-engineered, pre-finished  Trellis system.

  - RELATED SECTIONS

    - Section 03100 - Concrete Forms and Accessories.

    - Section 03300 - Cast-In-Place Concrete

    - Section 10530 – Protective  Covers

  - REFERENCES

    - American Architectural Manufacturers Association (AAMA):

      - AAMA 603 - Voluntary Performance Requirements and Test Procedures for Pigmented Organic Coatings on Extruded Aluminum.

      - AAMA 605 - Voluntary Specification for High Performance Organic Coatings on Architectural Extrusions and Panels.

      - AAMA 607.1 - Voluntary Guide Specification and Inspection Methods for Clear Anodic Finishes for Architectural Aluminum.

      - AAMA 608.1 - Voluntary Guide Specification and Inspection Methods for Electrolytically Deposited Color Anodic Finishes for Architectural Aluminum.

  - DESIGN REQUIREMENTS

    - Columns and Beams:  Aluminum extrusions.

    - Structural Framing:

      - Heli-arc welded, support columns and beams forming one-piece rigid bents.

      - Mechanically fastened support columns and beams using internally concealed bolted connections.

    - Building Code:  ____________________________.

    - Design Loads:

      - Comply with Building Code for site location.

      - Collateral Loads:  Additional loads imposed by other materials or systems identified in contract documents.

    - Structural Design:  Prepare complete structural design calculations for members except footings.  Provide reactions as required for footing design by a registered professional engineer.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data:  Manufacturer's catalog data, detail sheets, and specifications.

    - Shop Drawings:  Layout and erection drawings showing framing, cross sections, and details, clearly indicating proper assembly.

    - Samples:  Color selection samples consisting of actual coating material or anodizing process on aluminum extrusions.

    - Quality Assurance/Control Submittals:

      - Qualifications:  Letter certifying manufacturer's required qualifications.

      - Structural Design Calculations.

      - Manufacturer's Installation Instructions.

  - QUALITY ASSURANCE

    - Overall Standard:  Structural engineering design documents stamped by a structural engineer registered to practice in __________________________________.

    - Manufacturer Qualifications:  Minimum five years experience in producing Pergola and Trellis structures and structures of the type specified.

    - Installer Qualifications:  Minimum two years experience in erecting aluminum structures of the type specified.

  - DELIVERY, STORAGE, AND HANDLING

    - Conform to Section 01660 - Product Storage and Handling Requirements.

    - Follow manufacturer's instructions.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:  Perfection Architectural Systems, Inc., 2310 Mercator Drive Orlando, FL  32807; ASD. Tel: (800) 238-7207, Fax: (407) 671-8252.

    - Representative:   ________________________________.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

    - Substitutions:   Not permitted.

    - Provide all components from a single manufacturer.

  - MATERIALS

    - Aluminum Extrusions:  6063 alloy, T-6 temper.

    - Grout:  1 part portland cement, 3 parts masonry sand; 2,000 pounds per square inch (13.8 MPa) compressive strength.

    - Foam Block-Outs:  Rigid foam blocks sized as required for column embedment depth and shape.

  - COMPONENTS

    - Columns:

      - Radius-cornered aluminum tubular extrusion of size shown on drawings.

      - Radius-cornered aluminum tubular extrusion as required by structural engineering design.

      - Grout Key:  Provide two 1-1/2 inch (38 mm) diameter holes in column base, one each in opposite sides.

      - Provide clear acrylic protection coat on surfaces in contact with grout.

    - Support Beams:  Aluminum tubular extrusions with welded plate end closures and profile as indicated on drawings.

      - Size:  As shown on drawings.

      - Size:  As required by structural engineering design.

    - Cross Beams: Aluminum tubular extrusions with welded plate end closures shape and profile as indicated on drawings.

      - Size:  As shown on drawings.

      - Size:  As required by structural engineering design.

  - ACCESSORIES

    - Fasteners:

      - Screws:  No. 14 by 1 inch (25 mm), self-tapping, Type 18-8 stainless steel with neoprene washers.

      - Other Fasteners:  Type 18-8 stainless steel, type recommended by manufacturer for specific condition.

  - FABRICATION

    - Shop Assembly:  Fabricate support beams and columns into one-piece rigid bents with corners mitered and heli-arc welded to the extent that completed bents can be shipped on local, state, and federal highways without special permit.  Provide bolted connections for bents required to be shipped unassembled.

    - Shop Assembly:  Fabricate cross beams for field assembled mechanical connections.

  - FINISHES

    - Columns:

      - Clear Anodized:  AA-M-10C-22A-31, Architectural Class II, comply with AAMA 607.1.

      - Bronze Anodized:  AA-M-10C-22A-44, Architectural Class I, comply with AAMA 608.1.

      - Thermo-Set Enamel:  AA-C-12C-42R-1, comply with AAMA 603.

        - Color:  As selected by architect from manufacturer's standard color range.

        - Color:  Custom color as selected by architect.

      - Fluoropolymer Coating:  70 percent PVDF resin based fluoropolymer, AA-C-12C-42R-1, custom color as selected by architect, comply with AAMA 605.

        - Two coat application.

        - Three coat application.

    - Support Beams:

      - Clear Anodized:  AA-M-10C-22A-31, Architectural Class II, comply with AAMA 607.1.

      - Bronze Anodized:  AA-M-10C-22A-44, Architectural Class I, comply with AAMA 608.1.

      - Thermo-Set Enamel:  AA-C-12C-42R-1, comply with AAMA 603.

        - Color:  As selected by architect from manufacturer's standard color range.

        - Color:  Custom color as selected by architect.

      - Fluoropolymer Coating:  70 percent PVDF resin based fluoropolymer, AA-C-12C-42R-1, custom color as selected by architect, comply with AAMA 605.

        - Two coat application.

        - Three coat application.

    - Cross Beams:

      - Clear Anodized:  AA-M-10C-22A-31, Architectural Class II, comply with AAMA 607.1.

      - Bronze Anodized:  AA-M-10C-22A-44, Architectural Class I, comply with AAMA 608.1.

      - Thermo-Set Enamel:  AA-C-12C-42R-1, comply with AAMA 603.

        - Color:  As selected by architect from manufacturer's standard color range.

        - Color:  Custom color as selected by architect.

      - Fluoropolymer Coating:  70 percent PVDF resin based fluoropolymer, AA-C-12C-42R-1, custom color as selected by architect, comply with AAMA 605.

        - Two coat application.

        - Three coat application.

- EXECUTION

  - EXAMINATION

    - Examine footings in which columns will be set.  Verify footing locations and elevations comply with shop drawings.

    - Examine building surfaces to which structure will connect.

    - Coordinate with responsible trade to perform corrective work on unsatisfactory footings or surfaces.

    - Commencement of work by installer is acceptance of existing conditions.

  - ERECTION

    - Erect in accordance with manufacturer's installation instructions.

    - Set columns straight, and true to line, adequately braced to maintain position until grout has cured.

    - Keep aluminum surfaces from direct contact with ferrous metal or other incompatible materials by applying one coat of clear acrylic coating.

  - CLEANING

    - Clean surfaces soiled by work as recommended by manufacturer.

    - Remove surplus materials and debris from the site.

  - PROTECTION

    - Protect finished aluminum surfaces from damage due to subsequent construction operations.

END OF SECTION